| Feature or Option | How to use: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Receiving Calls | A call ringing at your extension | - Lift the receiver or press Speaker Converse |  |  |  |  |
| ### Call Pick Up | Answering your ringing phone from another phone | - When you hear your phone ringing, lift the receiver or press Speaker - Dial 67 followed by your extension number |  |  |  |  |
| Calling out | To another phone in the office | - Lift the receiver or press Speaker - Dial the extension number Note: this activates the speaker on the telephone you are calling, press 1 to make the telephone ring. |  |  |  |  |
|  | Out of the office | - Lift the receiver or press Speaker - Dial 9 and the number you are calling |  |  |  |  |
|  | Last Number Redial | - Press Redial, Press  |  |  |  |  |
|  | Using the system or Station speed dial | - Lift the receiver or press Speaker - Press the Redial button - Enter the 2 digit code assigned to the number you are calling (00 – 99) # OR - Press the triangle button under the display that has SYS (System) or STA (Station) - Press the Up or Down buttons to scroll between the names of the items in the list. - When you see the name you wish to call Lift the receiver or press Speaker |  |  |  |  |
| Speaker | - To turn you micro-phone on/off, press Feature 1 |  |  |  |  |  |
| Receiver Volume | - While on a call, - Adjust using the oblong arrow button beside Speaker |  |  |  |  |  |
| Change Contrast | - While phone is idle press the oblong arrow button beside speaker - Note: if you have back ground music on shut it off to adjust contrast |  |  |  |  |  |
| Back ground music On/Off | - Press Feature - Dial 26 |  |  |  |  |  |
| Ringing Volume Control | - Press Speaker - Dial 601 - Adjust using the oblong arrow button beside Speaker - Press Speaker to set |  |  |  |  |  |
| Transfer Calls | - With call in progress, press Transfer - Dial the station Number - Stay on the line to announce the call (optional), Hang up |  |  |  |  |  |
| Transferring calls to an external number, Using TELUS Transfer | - While on a call press RECALL, You will hear dial tone - Dial the number you want to transfer to - When the phone begins to ring you can hang up or stay on the line to announce the transfer - Press RECALL – this sets up a 3 way call - Hang up to remove yourself from the call and leave the 2 callers connected Note: if you reach a busy or are unable to complete the transfer press RECALL slowly twice to return to the original call |  |  |  |  |  |
| ##### Feature | How to use |  |  |  |  |  |
| Conference Call | Add an Internal call - With a call in progress, press Conf. - Call the extension you wish to add, - Press Conf to add the first caller |  | Add an External Call - With a call in progress, press Conf. - Dial 9 and the number you are calling, when they answer - Press Conf to add the first caller |  |  |  |
| PAGE | Lift Receiver : Dial 51(all sets only) |  | Dial 55 (External speaker only) |  |  | Dial 59 –(All sets & speakers) |
| ### One Set | - Lift receiver, Dial the extension number, NOTE: this activates the speaker, to make the phone ring dial 1 after the extension number |  |  |  |  |  |
| Call forward your extension to another NEC phone: | Set: Leave receiver down - Press Feature - Dial 60 - Enter forward to extension - Press Feature |  |  |  | To remove: Leave receiver down - Leave receiver down - Press Feature - Dial 69 - Press Feature |  |
| Do not Disturb | Set: Leave receiver down - Press Feature - Dial 60 - Press Feature |  |  |  | Remove: Leave receiver down - Press Feature - Dial 69 - Press Feature |  |
| Leave a message for another employee | - Dial the station number - Press 7 - Leave your message Note: To bypass listening to the complete greeting press  |  |  |  |  |  |
| Program Personal Speed Dial (RECEIVER DOWN) | - Press: Feature, - Press: Redial - Dial Speed Dial memory Location 80 – 99 - Dial Trunk Access Code: 9 Dial the number to store - Press HOLD (if entering the name) Press: Feature |  |  |  |  |  |
| ## Program Feature access Keys or Buttons | Outside Numbers - Press Feature - Press Redial - Press Button you want to program - Dial 0 9 the number you are storing - Press Feature |  |  | Internal Numbers - Press Feature - Press Redial - Press Button you want to program - Dial 1, Dial Extension Or Feature code to store - Press Feature |  |  |
| To program Names in Speed Dial: - Press the digit that has the desired letter on it - Eg: for N, press 6 twice - The codes will automatically advance to the next space but if you need two letters that are on the same number press the # button to advance to the next character. - Eg: for CAT: press 222 ( C),# 2(A) 8 (T) Press Conf if an error is made to back up a space |  |  |  |  |  |  |
| CCSI 104-1000 100 Ave Grande Prairie PH: 780-538-2012 www.ccsigp.ca FAX 780-539-5904 |  |  |  |  |  |  |